Notes by R.N. Folsom, 26 November 1990

DSKBENCH.EXE is a public domain disk testing routine, written by John

Navas, that tests hard disk speed.

I obtained DSKBENCH.EXE from the Bavug bulletin board, (415)574-7914,

Franz Hirner, sysop.

R.N. Folsom

128 Seafoam Avenue

Monterey, California  93940

(408)649-6383 or (408)924-5418 or -5400

John Navas' Original Documentation:

The DSKBENCH disk benchmark program uses only documented DOS calls

and so will run on ANY MS-DOS microcomputer.  The current drive is

always tested and there must be at least 360 KB of space available

for the tests to be run.  Best results are obtained with an empty

or defragmented disk.

Revision history:

1.2	Ensures that the I/O buffer does not cross a 64KB memory

boundary (which can adversely affect performance).

1.3	Corrects a bug in the buffer allocation routines

introduced in Version 1.2.

1.4	Improved error-handling.

1.5	General clean-up.

1.6	Ensures that the temporary file is always located on the

default drive.

1.7	Restarts timing if a sector read error occurs instead of

aborting.

The following results may be used for comparative purposes.  They

were obtained under IBM PC-DOS 2.10 with FILES=22 and BUFFERS=22.

The interleave was 2:1 on the Seagate ST-225 hard disk.

(Note that the processor is an 8086, not an 8088.)

4.77/9.54 MHz 8086, 360 KB Shugart diskette drive

Block  I/O    read   20.0 kb/sec.    write   13.8 kb/sec.

Sector I/O    read    2.4 kb/sec.    write    1.3 kb/sec.

DOS random sector read  204.3 ms. avg.

Effective rotation speed    300 rpm

4.77 MHz 8086, OMTI 5510 controller with Seagate ST-225

Block  I/O    read  141.1 kb/sec.    write  140.7 kb/sec.

Sector I/O    read   25.6 kb/sec.    write   13.4 kb/sec.

DOS random sector read   83.9 ms. avg.

Effective rotation speed   3602 rpm

9.54 MHz 8086, OMTI 5510 controller with Seagate ST-225

Block  I/O    read  148.0 kb/sec.    write  148.0 kb/sec.

Sector I/O    read   27.4 kb/sec.    write   14.5 kb/sec.

DOS random sector read   82.2 ms. avg.

Effective rotation speed   3602 rpm

4.77 MHz 8086, Lotus-Intel-Microsoft EMS ramdisk

Block  I/O    read  312.5 kb/sec.    write  268.2 kb/sec.

Sector I/O    read   48.3 kb/sec.    write   34.8 kb/sec.

DOS random sector read    4.4 ms. avg.

Effective rotation speed  16240 rpm

9.54 MHz 8086, Lotus-Intel-Microsoft EMS ramdisk

Block  I/O    read  521.6 kb/sec.    write  456.5 kb/sec.

Sector I/O    read  120.4 kb/sec.    write   82.7 kb/sec.

DOS random sector read    2.0 ms. avg.

Effective rotation speed  35714 rpm

9.54 MHz 8086, FLASH EMS ramdisk (no BUFFERS)

Block  I/O    read  386.1 kb/sec.    write  343.0 kb/sec.

Sector I/O    read  104.3 kb/sec.    write   60.3 kb/sec.

9.54 MHz 8086, FLASH EMS cache of OMTI 5510/ST-225 (no BUFFERS)

Block  I/O    read  380.1 kb/sec.    write  262.2 kb/sec.

Sector I/O    read  100.5 kb/sec.    write   26.2 kb/sec.

DOS random sector read   84.9 ms. avg.

Effective rotation speed  16599 rpm

Other reported results:

IBM PC w/Miniscribe 20 MB Hard Drive

Block  I/O    read   64.9 kb/sec.    write   36.1 kb/sec.

Sector I/O    read   22.5 kb/sec.    write   12.3 kb/sec.

DOS random sector read  118.0 ms. avg.

Effective rotation speed   3602 rpm

Leading Edge D V20 7.4 MHz DTC 5160 RLL Seagate ST-238 (3:1 interleave)

Block  I/O    read  174.0 kb/sec.    write  177.6 kb/sec.

Sector I/O    read   26.9 kb/sec.    write   15.2 kb/sec.

DOS random sector read   84.0 ms. avg.

Effective rotation speed   3600 rpm

Leading Edge D V20 7.4MHz DTC 5160 RLL Seagate ST-277R (3:1 interleave)

Block  I/O    read  212.0 kb/sec.    write  213.2 kb/sec.

Sector I/O    read   27.5 kb/sec.    write   15.3 kb/sec.

DOS random sector read   36.1 ms. avg.

Effective rotation speed   3598 rpm

Leading Edge D V20 7.4MHz LIM spec ramdisk on JRAM-3 card.

Block  I/O    read  593.0 kb/sec.    write  476.6 kb/sec.

Sector I/O    read   76.1 kb/sec.    write   58.1 kb/sec.

DOS random sector read    2.8 ms. avg.

Effective rotation speed  24541 rpm

Victor 9000 w/Tandon 100-4 1.2MB Floppy Drive

Block  I/O    read   20.9 kb/sec.    write    8.4 kb/sec.

Sector I/O    read    2.2 kb/sec.    write    0.8 kb/sec.

DOS random sector read  470.0 ms. avg.

Effective rotation speed    252 rpm

Victor 9000 w/Tandon 10 MB Hard Drive

Block  I/O    read   76.7 kb/sec.    write   76.7 kb/sec.

Sector I/O    read   13.1 kb/sec.    write    9.2 kb/sec.

DOS random sector read  117.1 ms. avg.

Effective rotation speed   1800 rpm*

Victor 9000 w/30 MB Rodime Hard Drive

Block  I/O    read   72.1 kb/sec.    write   81.9 kb/sec.

Sector I/O    read   13.1 kb/sec.    write    9.0 kb/sec.

DOS random sector read   86.7 ms. avg.

Effective rotation speed   1800 rpm*

Victor 9000 w/Priam VT-150 40 MB. Hard Drive

Block  I/O    read   72.1 kb/sec.    write   80.1 kb/sec.

Sector I/O    read   13.4 kb/sec.    write    9.2 kb/sec.

DOS random sector read   50.0 ms. avg.

Effective rotation speed   1800 rpm*

Victor 9000 w/Atasi 40 MB Hard Drive

Block  I/O    read   76.7 kb/sec.    write   75.1 kb/sec.

Sector I/O    read   13.4 kb/sec.    write    9.4 kb/sec.

DOS random sector read   53.3 ms. avg.

Effective rotation speed   1800 rpm*

Victor 9000 w/NEC & Tandon 703 30Mb Drive:

Block  I/O    read   76.7 kb/sec.    write   90.1 kb/sec.

Sector I/O    read   14.1 kb/sec.    write    9.4 kb/sec.

DOS random sector read   56.0 ms. avg.

Effective rotation speed   1800 rpm*

* NOTE: Since these drives actually rotate at 3600 rpm, it appears

that the Victor always loses a revolution when rereading the same

sector.

Victor 9000 PLUSPC & Cogito 912 10Mb Drive:

Block  I/O    read   78.5 kb/sec.    write   74.9 kb/sec.

Sector I/O    read   23.0 kb/sec.    write    9.2 kb/sec.

DOS random sector read  200.0 ms. avg.

Effective rotation speed   3649 rpm

Victor VI (IBM Mode) w/Rodime 30 Meg.

Block  I/O    read   71.4 kb/sec.    write   71.2 kb/sec.

Sector I/O    read   21.7 kb/sec.    write    9.1 kb/sec.

DOS random sector read   78.7 ms. avg.

Effective rotation speed   3556 rpm

Victor VI (Victor Mode) w/Rodime 30 Meg.

Block  I/O    read   70.7 kb/sec.    write   69.8 kb/sec.

Sector I/O    read   21.8 kb/sec.    write    9.1 kb/sec.

DOS random sector read   78.7 ms. avg.

Effective rotation speed   3546 rpm

Victor 9000 w/ White Crane RAMDISK (500K)

Block  I/O    read  270.3 kb/sec.    write  206.0 kb/sec.

Sector I/O    read   73.2 kb/sec.    write   63.2 kb/sec.

DOS random sector read    2.3 ms. avg.

Effective rotation speed  38250 rpm

Victor 9000 w/WA Escort 1.5 Board RAMDRIVE

Block  I/O    read  468.6 kb/sec.    write  439.6 kb/sec.

Sector I/O    read   74.2 kb/sec.    write   54.9 kb/sec.

DOS random sector read    2.7 ms. avg.

Effective rotation speed  27000 rpm

PC DESIGNS FD1000 NEC V-20 8MHz WD1002 & Seagate ST-225 (4:1 interleave)

Block  I/O    read   77.0 kb/sec.    write   77.0 kb/sec.

Sector I/O    read   22.7 kb/sec.    write   13.0 kb/sec.

DOS random sector read  109.5 ms. avg.

Effective rotation speed   3604 rpm

PC DESIGNS FD1000 NEC V-20 8MHz OMTI 5520 & Toshiba MK56 (2:1 interleave)

Block  I/O    read  218.8 kb/sec.    write  220.0 kb/sec.

Sector I/O    read   27.5 kb/sec.    write   14.4 kb/sec.

DOS random sector read   32.3 ms. avg.

Effective rotation speed   3604 rpm

"Brand X" 80386 w/30 MB Rodime Hard Drive

Block  I/O    read  141.6 kb/sec.    write  139.2 kb/sec.

Sector I/O    read  152.0 kb/sec.    write   25.5 kb/sec.

DOS random sector read   66.3 ms. avg.

Effective rotation speed   3574 rpm