Documentation of Sirius Codec Circuits: "Experimenters' Notes"

--------------------------------------------------------------

1. Main Board (Motherboard)

---------------------------

Audio input is direct to the 55516-9 Codec chip only passing

through DC blocking capacitor C30. This capacitor, being low in

value, will almost certainly also degrade HF performance

depending on the input impedance of the analogue input of the

codec chip. Input level should not exceed 2V peak-to-peak =

0.707V RMS = 700mV RMS. Take care not to overload the input. Op-

amps with inputs 2,3 and 5,6 of the motherboard LM324 are a

Butterworth active filter with 3KHz low-pass characteristic. The

only function of the op-amp with pins 9 and 10 as inputs is to

provide bias; it supplies an accurate, low-impedance 6V source

for biasing on the Codec Preamp which it provides via pin 6 of

J17, the connector to the Codec Preamp Board. In addition it

appears to be used during the off periods of the 4066 quad

bilateral switch. This device is used to vary sound volume by

alteration of the duty cycle and the bias is probably used here

to help eliminate switching noise.

2. First Codec Preamp

---------------------

The first half of the LM358, with input pins 2 and 3, is used as

an amplifier and compressor/limiter. The limit function is

accomplished by a constant bias at the gate of FET 2N5459 from

the motherboard and a variable bias from the second op-amp via C7

and R12. A large output voltage at the output of the second op-

amp with pins 5 and 6 as input will reduce the gain of the input

op-amp by the FET in its feedback loop, thus limiting the output

signal level to one which is safe for input to the Codec at pin

5. The FET 2N5459 is readily obtainable.

2. Second Codec Preamp

----------------------

The first op-amp has a gain of 33 and also has a compressor/limit

function performed by half of an analogue opto-isolator, the

CLM6200 which is unobtainable. The degree of limiting by

alteration of the gain of the input stage is determined by the

side chain beginning with the charge pump R8, C4, C7. Another

part of the LM324, with input pins 12 and 13, is used as a sense

amplifier and the current passed through the collector of Q1

feeds back via the LED in the CLM6200 to the feedback loop of the

op-amp at the input. VR1 sets the overall gain and is adjusted

for 2V p-p (700mV rms) at the output without limiting taking

place. After any limiting at the first op-amp the signal is given

a further gain of around 10 by the op-amp with feedback resistor

R6 and input pins 5,6. The final stage amplifier with input pins

9,10 gives the signal a treble boost before being output to the

motherboard at J1 pin 5.

PAGE 1 OF 2  11/12/92  ISSUE, Postbus 3707, 1001 AM Amsterdam, NL

From the Sirius Hardware Reference Manual page 109 (First Preamp)

-----------------------------------------------------------------

INPUT SIGNAL CONDITIONING

The microphone amplifier utilizes half of an LM358 and a JFET in

a variable-gain amplifier used as a compressor. The attack time

of the compressor is about 50 milliseconds; release time is 250

mS. Input signal amplitude range for acceptable record quality is

about 5 to 75 mV RMS. The second stage, 1/2 of a LM358, is a 3-

pole butterworth low-pass filter with a cutoff-frequency of about

3KHz. The filter eliminates "aliasing" in the CVSD modulator.

OUTPUT CONDITIONING AND POWER AMPLIFIER

Following the CVSD, the output (playback) signal is low-pass

filtered by another active, 3KHz cutoff butterworth filter (1/4

LM324). Following this stage, a CA4066B and its attendant drivers

provide software-controlled volume control by varying the duty-

factor of signal CODEC VOL. The frequency of this signal

(including the produced sidebands) must be high enough to be

above audible range; a minimum of 20KHz is recommended. Playback

power amplification is provided by an LM383. This stage also

provides some roll-off to alleviate the above problem. The power

stage will produce 4 watts of audio; thus, an external speaker

should be used if above-normal sound levels are programmed, since

the internal speaker is rated at only 300 milliwatts.

Additional Notes on the Second Preamp Circuit

---------------------------------------------

This is the most advanced preamp circuit of the two and might

still be viable if a readily available substitution for the

CLM6200 can be found. Suggested types might be the H11F1 or H11F3

Bilateral Analogue FET Output types by Harris or, possibly less

likely, the Siemens SF4 620-2 optocoupler (all available from

Farnell). Some or all of these opto-couplers should be usable in

the second preamp circuit with some alteration of the circuit

resistor values.

From the Victor 9000 Maintenance Manual page 3-85:

--------------------------------------------------

Section 3.4.16: CODEC CODER/DECODER

The Victor 9000 digitizes voice through the use of a codec.

The codec (HV 55516) uses the CVSD (CONTINUOUSLY VARIABLE SLOPE

DELTA MODULATION) method for digitizing analog signals and for

converting digital information into audio signals.

CVSD is a simple way to digitize analog signals. The analog

signal is input to the CVSD device. The signal is then applied to

a comparator and an integrator. The output of the integrator is

applied as the second input to the comparator to form a feedback

loop. The output of the comparator is the sign difference between

the audio input and the integrator output. The sign bit is also

used to control the gain of the integrator by varying the

direction of the ramp of the integrator.

PAGE 2 OF 2  11/12/92  ISSUE, Postbus 3707, 1001 AM Amsterdam, NL

###################################################