SUPER-BIOS REFERENCE

--------------------

Super-BIOS Interface Description

Setup:    AX = Super-BIOS function number

ES:BX = long pointer to parameter

block

Invoked:  Software interrupt 223 (DF hex)

Return:   As specified in function description.

Stack and Code environment is preserved.

Super-BIOS interface requires three

words of caller's stack.

DS register is preserved.

AX,ES, and BX are reserved for

return values.

Function Number          Description

_______________          ___________

0   Super-BIOS Release Number

Returns a unique number for identifying

the Super-BIOS release level.

parm1 = release number        (word/output)

1   Get Zone (primarily for diagnostics use)

Converts a track number to a zone number.

parm1 = track number          (word/input)

parm2 = zone number           (word/output)

2   Get Sectors Per Track (primarily for diagnostics)

Returns the number of sectors

for a specified track.

parm1 = track number          (word/input)

parm2 = number of sectors     (word/output)

3   Allocate Disk Job (primarily for diagnostics)

Returns a pointer to disk job structure.

(See listing of BIOS structurs.)

parm1 = long pointer to job structure

(long word/output)

4   Start Disk Job (primarily for diagnostics)

Submit a job to the low-level disk driver.

parm1 = long pointer to job structure

(long word/input)

parm2 = priority              (word/input)

5   Get Drive Information (primarily for diagnostics)

Return a pointer to a drive's information

structure.  (See listing of BIOS structures.)

parm1 = drive number          (word/input)

parm2 = long pointer to drive info structure

(long word/output)

6   Read Sector

Read physical sector(s) into memory with

full retries.

parm1 = drive  number         (word/input)

parm2 = track  number         (word/input)

parm3 = sector number         (word/input)

parm4 = sector count          (word/input)

parm5 = long pointer to memory buffer

(long word/input)

7   Write Sector

Write physical sector(s) from memory with

sector verify and full retries.

parm1 = drive  number         (word/input)

parm2 = track  number         (word/input)

parm3 = sector number         (word/input)

parm4 = sector count          (word/input)

parm5 = long pointer to memory buffer

(long word/input)

8   Convert Logical Sector to Physical Disk Address

Convert an absolute sector number into

a physical track and sector number.

parm1 = drive number          (word/input)

parm2 = logical sector number (word/input)

parm3 = interleave factor     (word/input)

parm4 = track number          (word/output)

parm5 = sector number         (word/output)

9   Flush Buffers                 (Not Implemented)

Flush all updated ("dirty") bufers to disk.

parm1 = drive number          (word/input)

10   Display Disk Error

Display BIOS error message for last disk

operation.

11   Get Character Generator            (Not Implemented)

Returns the length and the starting address

of character generator dots in memory.

parm1 = byte count            (word/output)

parm2 = base address of dots  (word/output)

12   Set Character Generator Size       (Not Implemented)

Set the size of the character generator.

parm1 = byte count            (word/input)

13   Get Screen Address                 (Not Implemented)

Return the location of screen memory.

parm1 = base of screen        (word/output)

14   Get Device Vector

Get the current vector for the specified

device driver.

parm1 = device number         (word/input)

parm2 = device vector         (long word/output)

Device Vectors:

0   TOD Vector      Timer (called every .5 sec.)

1   CON Vector      Console (keyboard & display)

2   LST Vector      Listing device (printer)

(LST longer available in DOS 2.11 BIOS 2.9)

Functions when calling Vectors 1 and 2 (AX = func)

0   INPUT   Character is returned in AL.

1   OUTPUT  Character in CL is sent.

2   INSTAT  AL = 0: no data,  AL = FF: data ready

(if data ready then data is in AH)

3   OUTSTAT AL = 0: busy,     AL = FF: ready

4   GET DEVICE VECTOR  Returns in ES:BX

LST input vector or User Defined Console vect.

5   SET DEVICE VECTOR  (Does not work)

15   Set Device Vector

Set the vector for the specified device

driver.

parm1 = device number    (word/input)

parm2 = device vector    (long word/input)

16   Get I/O Byte

Returns the I/O byte.    (removed in 2.11)

parm1 = I/O byte         (word/output)

17   Set I/O Byte

Set the I/O byte.        (removed in 2.11)

parm1 = I/O byte         (byte/input)

18   KBCHRHK  (Keyboard and Character set hooks.)

Table function call:

parm1 = 0:  Set Character Set  (byte/input)

= 1:  Get Character Set

= 2:  Set Keyboard Table

= 3:  Get Keyboard Table

= 4:  Set Xlate Table

= 5:  Get Xlate Table

parm2 = bytes to process       (word/input/output)

parm3 = pointer to user buffer (long word/input)

INTERNAL BIOS STRUCTURES

------------------------

DIB       struct        ; DRIVE INFORMATION STRUCTURE

secsize   dw    ?       ; bytes per sector

bsecs     db    ?       ; boot sector, count

secleave  db    ?       ; interleave factor

side2     db    ?       ; 2 sided disk flag

motoroff  db    ?       ; motor turn off time

ledoff    db    ?       ; LED turn off time

currtrk   db    ?       ; curr track #

currsec   db    ?       ; curr sector on track

priority  db    ?       ; curr priority level

state     db    ?       ; curr state in sm

jobcnt    db    ?       ; of jobs running

t0        db    ?       ; temp

job       dd    ?       ; curr job

timeout   db    ?       ; time-out counter

seekcnt   db    ?       ; retries for seeking

steps     dw    ?       ; remaining steps

doorsense db    ?       ; state of door

readonly  db    ?       ; drive read-only flag

DIB       ends

JDISK     struc         ; DISK JOB STRUCTURE

status    db    ?       ; job state and priority

func      db    ?       ; job type

drive     db    ?       ; drive#: 0 or 1

track     db    ?       ; track# and side bit

sector    db    ?       ; sector on track

seccnt    db    ?       ; # of sectors to read

dma       dd    ?       ; buffer address

errcode   db    ?       ; completion code

errsens   db    ?       ; further error info

errsec    db    ?       ; sector in error

errcnt    dw    ?       ; sectors remaining

JDISK     ends

declare     DLABEL      structure(

labelid     word,           /* label id: ff = boot */

loadaddr    word,            /* boot code load seg */

loadlen     word,       /* boot size in paragraphs */

entry       pointer,           /* boot entry point */

partid(8)   byte,          /* diskette part number */

serial(8)   byte,        /* diskette serial number */

ssize       word,          /* sector size in bytes */

dstart      word,          /* first sector of data */

boot        word,          /* first sector of boot */

flags       word,   /* interleave and double sided */

disktype    byte,              /* =0 for cpm disks */

reserved(3) byte,          /* for future expansion */

speed(18)   byte,            /* speed control info */

zones(15)   byte,                   /* zoning info */

spt(15)     byte,             /* sectors per track */

extra(44)   byte       /* remaining part of sector */

)

DISK TABLES

side0 equ 4*19+12*18+11*(17+16)+10*15+12*14+11*13+9*12

; sectors = 04C8h

side1 equ 8*18+11*(17+16)+10*15+12*14+11*13+12*12+5*11

; sectors = 048Fh

disktbls db     4       ;        number of drivers

db      0       ;        drive a

dw      dpt10   ;        ..single sided

db      0       ;

dw      dpt20   ;        ..double sided

db      1       ;        drive b

dw      dpt10   ;        ..single sided

db      1       ;

dw      dpt20   ;        ..double sided

; SINGLE SIDED DISK

dpt10   dw      512     ;        sec size

db      4       ;        clu size

dw      1       ;        boot secs - label only

db      2       ;        allocation tables

dw      128     ;        directory entries

dw      side0   ;        sectors

; DOUBLE SIDED DISK

dpt20   dw      512     ;        sec size

db      4       ;        clu size

dw      1       ;        boot secs - label only

db      2       ;        allocation tables

dw      128     ;        directory entries

dw      side0+side1     ;sectors

ASYNC INTERFACE BLOCK

SCHEDULE WORK TO INTERRUPT HANDLER

aibstr  struc

AIBlock   db    ?       ; syncronization lock

pending   db    ?       ; pending flag

func      db    ?       ; type of job

trk       db    ?       ; target track

sec       db    ?       ; target track

seccnt    db    ?       ; # sectors to be read

secsize   dw    ?       ; bytes per sector

retries   db    ?       ; retry counter

syncretry db    ?       ; retry counter

retcode   db    ?       ; completion code

dma       dd    ?       ; pointer,buffer addr in ram

timer     dd    ?       ; pointer,timer addr

AIBstr    ends

FLOPPY DISK JOB STATES

f$FREE        equ 0     ;    job free to be allocated

f$PENDING     equ 1     ;       job allocated to user

f$RUNNING     equ 2     ;  job accepted by state mach

f$DONE        equ 3     ;        job returned to user

FLOPPY DISK JOB TYPES

f$READ        equ 1     ;    read one or more sectors

f$WRITE       equ 2     ;   write and verify a sector

f$VERIFY      equ 3     ; verify a sector against ram

f$SEEK        equ 4     ;             seek to a track

f$READHDR     equ 5     ;          read sector header

f$RESTORE     equ 6     ;             bump to track 0

f$STEPIN      equ 7     ;           step in one track

f$STEPOUT     equ 8     ;          step out one track

f$QWRITE      equ 9     ;        write without verify

f$MAXJOB      equ 9     ;              last valid job

f$SIDEBIT     equ 80h   ;         side bit in track #

f$TRKOFFSET   equ 8     ;     track offset for side 2

FLOPPY DISK ERROR CODES

f$OKAY        equ 0     ;          no error condition

f$NOSYNC      equ 0ffh  ;      no sync found on track

f$JOBTIMEOUT  equ 0feh  ;           job took too long

f$SEEKERR     equ 0fdh  ;            track seek error

f$WRPROTECT   equ 0fch  ;     write to protected disk

f$MTRTIMEOUT  equ 0fbh  ;       motor not up to speed

DLABEL  -  Disk Label Handler

f$DOOROPEN    equ 0fah  ;             drive door open

f$BADJOB      equ 0f9h  ;             bad func number

f$BADDRIVE    equ 0f8h  ;            bad drive number

f$BADTRACK    equ 0f7h  ;            bad track number

f$NOJOBS      equ 0f6h  ;     job cannot be allocated

f$BADLABEL    equ 0f5h  ;       wrong disk label type

f$READONLY    equ 0f4h  ;     disk in read-only state

f$BUFFLOST    equ 0f3h  ;      dirty buff not written

f$STEPERR     equ 0f2h  ;            track 0 detected

f$SIDE2ERR    equ 0f1h  ;        cannot access side 2

f$ERRPRIORITY equ 3     ;          error job priority

FLOPPY DISK DRIVER STATE MACHINE

State  0:  get best track and turn on motor

State  1:  wait for motor speed to stabalize

State  2:  check if head stepping is required

State  3:  step single track

State  4:  terminate job if RESTORE,STEPIN,STEPOUT

State  5:  select drive, read header

State  6:  wait for header, check for wrong track

State  7:  get best job on track and start job

State  8:  wait for job to complete, set errcode

State  9:  turn off motor and disable state machine

declare   SYS   struc (

MRT (33)    BYTE,

iobyte      byte,

actrlsb     byte,

actrmsb     byte,

bctrlsb     byte,

bctrmsb     byte,

cr2a        byte,

cr4a        byte,

cr4b        byte,

cr3a        byte,

cr3b        byte,

cr5a        byte,

cr5b        byte,

disp$BANK   word,

logo$base   word,

disp$base   word,

blank       word,

ch$size     word,

ch$set$cnt  word,

end$parms   byte

)

Key Descriptor Definitions (KD__)

(2) Functions         (1) Legend           (0) Data

.__________________.  .__________________. .__________________.

| t t a l  c s L L |  | L L L L  L L L L | | D D D D  D D D D |

BYTE

0      Data           key code or index pointer.

1      Legend         8 LSBits of legend code.

2      Functions              key type,attributes,legend.

ttal csLL      (bits 7-0)

tt.. ....       TYPE:00 special functions

01 single data byte

10 multi-code data

11 (reserved for expansion).

..a. ....       AUTO-REPEAT this key.

...l ....       LOCAL, generate data direct to

display routine.

.... c...       CAPS LOCK, generate shifted value

when caps lock mode is on.

.... .s..       SHIFT LOCK, generate shifted value

when shift lock mode is on.

.... ..LL       2 MSBits of legend code

ATTRIBUTE BITS

declare   KD$auto$bit   lit '20h';

declare   KD$local$bit  lit '10h';

declare   KD$caps$bit   lit '08h';

declare   KD$shift$bit  lit '04h';

LOCAL KEY EVENT ENVIRONMENT

declare  KE0  struc (

KD0           byte,      /* key descriptor data field        */

KD1           byte,      /* key descriptor legend field      */

KD2           byte,      /* key descriptor functions field   */

index         word,      /* index into descriptor tables     */

key$hit       boolean,   /* flag: key hit active             */

shift$all     boolean,   /* flag: shift lock mode active     */

caps$on       boolean,   /* flag: caps lock mode active      */

mode$keys (1) boolean,   /* 00 flag: dead key active         */

caps$lock     boolean,   /* 01 flag: caps lock key active    */

shift$lock    boolean,   /* 02 flag: shift lock key active   */

left$shift    boolean,   /* 03 flag: left shift key active   */

right$shift   boolean,   /* 04 flag: right shift key active  */

alternate     boolean,   /* 05 flag: alt key active          */

control$mode  boolean,   /* 06 flag: control key active      */

repeat$mode   boolean,   /* 07 flag: repeat key active       */

clr$kb        boolean,   /* 08 flag: clear keyboard key      */

hold$line     boolean,   /* 09 flag: advance line key active */

hold$page     boolean,   /* 10 flag: advance page key active */

last$event    byte,      /* last event of non-function key   */

nextdata0     byte,      /* 1st byte of 1 or 2 char sequence */

nextdata1     byte,      /* 2nd byte of 2 char sequence      */

ch$ptr        ptr,       /* pointer to character data        */

ch$cnt        word,      /* character count of data          */

ch$index      word       /* ch$ptr index to current char     */

);

ACTIVE CONSOLE KEY EVENT ENVIRONMENT

declare  KE1  struc (

KD0           byte,      /* key descriptor data field        */

KD1           byte,      /* key descriptor legend field      */

KD2           byte,      /* key descriptor functions field   */

index         word,      /* index into descriptor tables     */

key$hit       boolean,   /* flag: key hit active             */

shift$all     boolean,   /* flag: shift lock mode active     */

caps$on       boolean,   /* flag: caps lock mode active      */

mode$keys (1) boolean,   /* 00 flag: dead key active         */

caps$lock     boolean,   /* 01 flag: caps lock key   active  */

shift$lock    boolean,   /* 02 flag: shift lock key active   */

left$shift    boolean,   /* 03 flag: left shift key active   */

right$shift   boolean,   /* 04 flag: right shift key active  */

alternate     boolean,   /* 05 flag: alt key active          */

control$mode  boolean,   /* 06 flag: control key active      */

repeat$mode   boolean,   /* 07 flag: repeat key active       */

clr$kb        boolean,   /* 08 flag: clear keyboard key      */

hold$line     boolean,   /* 09 flag: advance line key active */

hold$page     boolean,   /* 10 flag: advance page key active */

last$event    byte,      /* last event of non-function key   */

nextdata0     byte,      /* 1st byte of 1 or 2 char sequence */

nextdata1     byte,      /* 2nd byte of 2 char sequence      */

ch$ptr        ptr,       /* pointer to character data        */

ch$cnt        word,      /* character count of data          */

ch$index      word       /* ch$ptr index to current char     */

)

---------------------------------------------------------------

########################################################################