VICTOR 9000 INTERRUPT VECTOR TABLE

Interrupt            Function

Number

HEX     DEC

0       0            Divide by zero

1       1            Single Step

2       2            Non-maskable interrupt (NMI)

3       3            Break Point instruction

4       4            Overflow

5-17    5-23         Intel reserved

18-19   24-25        Date & Time continuously updated here

1A-1F   26-31        Intel reserved

20      32           DOS program terminate

21      33           DOS function call

22      34           DOS terminate address

23      35           DOS CTRL-BRK (^C) exit address

24      36           DOS fatal error vector

25      37           DOS absolute disk read

26      38           DOS absolute disk write

27      39           DOS terminate,fix in storage

28-3F   40-63        DOS reserved                     8259a PIC

4����   64           Sync detect hardware interrupt <----------

41      65           Serial communications hardware interrupt |

42      66           Time of day hardware interrupt           |

43      67           Interrupt to address all 6522 IRQs       |

44      68           Hard Disk                                |

45      69           Network                                  |

46      70           Keyboard hardware interrupt              |

47      71           Vertical sync hardware interrupt <--------

48      72           Network support routines

49-67   73-103       Reserved (VICTOR (not used))

68      104          Scan code interrupt (Bios Ver.2.93+)

69      105          Reboot interrupt (Bios Ver. 2.93+)

6A-7F   106-127      Reserved (VICTOR (not used))

80-BF   128-191      Reserved for user software interrupts

CO-CF   192-207      Reserved (VICTOR Diagnostics)

DO-D9   208-217      Reserved (VICTOR (not used))

DA-DB   218-219      GW Basic

DC      220          Audio Toolkit Basic Interface

DD-DE   221-222      IEEE 488

DF      223          Super Bios

EO      224          CP/M-86 and CP/M Emulator

E1-FF   225-255      Reserved (VICTOR (not used))

.pa�

VI ROM BIOS SERVICE ROUTINES

-------------------------------------------------------------------------------

Vecto�   Interrup�  Nam�                Entr�Labe�(wher� Ints

Address   Number                          IBM Mode     point to)

-------------------------------------------------------------------------------

00-03     0          Divide by zero        system

04-07     1          Single Step           system

08-OB     2          Nonmaskable           system

OC-OF     3          Breakpoint            system

10-13     4          Overflow              system

14-17     5          Print Screen          PRINT_SCREEN

18-1B     6          Reserved              DUMMY_INT_HANDLER

1C-1F     7          Reserved              DUMMY-INT-HANDLER

---------Hardware Service Routines---------------------------------------------

20-23     8          Time of Day Service   TIMER_INT_HANDLER

24-27     9          Keyboard Service      KB_INT_HANDLER

28-2B     A          Reserved              DUMMY_INT_HANDLER

2C-2F     B          Comms Service COM2:   DUMMY_INT_HANDLER

30-33     C          Comms Service COM1:   DUMMY_INT_HANDLER

34-37     D          Disk Service          HD_INT_HANDLER

38-3     E          Diskett� Servic�    DISKETT�_IN�_HANDLER

3C-3F     F          Printer Service       DUMMY_INT_HANDLER

-------------------------------------------------------------------------------

40-43     10         Video I/O             VIDEO_I/O

44-47     11         Equipment Check       EQUIPMENT_CHECK

48-4B     12         Memory                MEMORY_CHECK

4C-4F     13         Diskette/Disk I/O     DISK_IO

50-53     14         Communications        SERIAL_IO

54-57     15         Cassette              DUMMY_INT_HANDLER

58-5B     16         Keyboard I/O          KEYBOARD_IO

5C-5F     17         Printer               PARALLEL_IO

60-63     18         Resident BASIC        DUMMY_INT_HANDLER

64-67     19         Bootstrap             REBOOT

68-6B     1A         Time of Day           TOD

6C-6Ơ����1 ������  Keyboar�Brea�        DUMMY_INT_HANDLER_RET

70-73     1C         Timer Tick(18.2 /sec) DUMMY_INT_HANDLER_RET

74-77     1D         Video Initialisation  VIDEO_TABLE

78-7B     1E         Disk Operating Params FDISK_TABLE

7C-7F     1F         Video Graphics Params DUMMY_INT_HANDLER

100-103   40         XT Entry to Diskette  DISK_I/O

104-107   41         Fixed disk Parameters DISK_TABLE

10C-10F   43         Fixed disk Parameters DISK_TABLE

.pa�

VI ROM BIOS INTERFACE ROUTINES

----------------------------------------------------------------

10    ---------------VIDEO I/O----------------------------------

----------------------------------------------------------------

Intr  Func  In         Out   Comments

(AH)  Regs       Regs

---   0  Set CRT Mode

A̠              0=40x25 black&white 1=40x25 col

2=80x25 black&white 3=80x25 col

4=320x200 medium res colour

5=320x20� med res black&white

6=640x200 high res black&white

---   1  Set Cursor Type

C�               Star� sca� lin� (0-� C/G,0-1�mono) �

C̠              En� sca� lin� (se� star� t� 20Ƞ fo� �

no cursor)

---   2  Set Curson Position

DH.DL            Row column (0.0 = home)

BH               Video Page

---   3  Read Cursor Position

BH               Video Page

DH.DL   Row, column (0.0 = home)

---   4  Read Light Pen      Not supported

---   5  Select Active Page

A̠              New page (0-7 for 40x25 0-3for80x25)

---   6  Scroll Active Page Up

---   7  Scroll Active Page Down

A̠              Number of lines to scroll (0 if

entire screen)

CH,CL            Row, column of upper left corner

DH,DL            Row,column of lower right corner

BH               Attribute to use in blanked lines

---   8  Read attribute/character at current position

BH               Video page

AL     Character read

AH     Attribute of char (alpha modes only)

---   9  Write Attribute/character at current cursor position

BH               Video Page

CX               Count of characters to write

AL               Character to write

BL               Attribute to write

---   A  Write character only at current cursor position

BH               Video page

CX               Count of characters to write

AL               Characters to write

---   B  Set Colour Palette

BH               Palette colour ID being set

B̠              Colou�val to be used with col ID

---   C  Write dot

DX,CX            Row, column number

AL               Col value (if bit 7=1 value is OR'ed

---   D  Read dot

DX,CX            Row column number

AL     Dot read

.pa�

VI ROM BIOS INTERFACE ROUTINES

----------------------------------------------------------------

10 continued--------Video I/O-----------------------------------

----------------------------------------------------------------

Intr  Func  In        Out    Comments

(AH)  Regs      Regs

---   E  Write Teletype to Active Page

AL               Character to write

BL               Foreground colour in graphics mode

---   F  Get Current Video State

AL     Mode currently set (see func.0)

AH     Columns on screen

----------------------------------------------------------------

11-------------------Equipment----------------------------------

----------------------------------------------------------------

Intr  Func  In        Out    Comments

(AH)  Regs      Regs

None

AX     Equipment attached;Bits set as

follows;

A H               A L

0 0 X 0 0 0 0 X   0 0 0 0 0 0 X 0

---   - -----     --- --- ---   -

1    2   3        4   5   6    7

1 Numbers of printers

2 Game I/O

3 RS232 cards

4 Drives

5 Initial video mode

6 System board RAM

7 IPL from diskette

----------------------------------------------------------------

12   ---------------- Memory Size-------------------------------

----------------------------------------------------------------

Intr   Func  In         Out   Comments

(AH)  Regs       Regs

None

AX    Number of bytes in Kbytes

.pa�VI ROM BIOS INTERFACE ROUTINES

----------------------------------------------------------------

13   ---------------- Disk I/O

----------------------------------------------------------------

Intr   Func  In         Out    Comments

(AH)  Regs       Regs

---    0  Reset Disk System

None       None   Hard resets floppy and/or fixed

disk drives. Recal req'd

---    1  Read Disk Status from Last Operation

AL     Status byte;Bits set as follows;

01 Bad command passed to disk I/O

02 Address mark not found

03 Write attempt on protected disk

04 Requested sector not found

05 Reset failed

07 Drive parameter activity failed

08 DMA overflow on operation

09 Attempt to DMA across 64k bound

0B Bad track flag detected

10 Bad crc on diskette read

10 Bad ECC on disk read

11 ECC corrected data error

20 Controller has failed

20 NEC controller has failed

40 Seek operation failed

80 Attachment failed to respond

0BB Undefined error occurred

OFF Sense operation failed

The following registers are used for all read, write, and verify

operations��  Othe� non-absolut� command� mak� consisten� us� o� �

these registers as required.

AL                Number of sectors

CH                Track number for MFM floppy (0-39

Track number for GCR floppy 0-159

Cylinder no. for fixd disk 0-1023

CL                Sector number

Note; Upper 2 bits of cylinder no.

are placed in upper 2 bits

of CL

DH                Head number

DL                Drive number (0-3h for floppies &

80-87h for fixed disks)

ES:BX             Address of memory buffer to be

used for this operation

AH       Status of current operation

(values are defined above)

CY       1 Error in operation

0 Successful operation (AH returns

0)

---   2   Read the desired MFM sectors into memory

---   3   Write the desired MFM sectors from memory

---   4   Verify the desired sectors

---   5   Format the desired track (floppy only)

.pa�

VI ROM BIOS INTERFACE ROUTINES

----------------------------------------------------------------

13 continued-----Disk I/O---------------------------------------

----------------------------------------------------------------

Intr   Func   In      Out      Comments

(AH)   Regs    Regs

The following options are for the fixed disk only

---     6   Format the desired track bad - not supported

---     7   Format drive                 - not supported

---     8   Return drive parameters

CL       Numbers of sectors/track

CH       Numbers of cylinders (high 2 bit

in CL)

DL       Number of fixed disk drives

DH       Number of heads

---     9   Initialize drive parameters

---     A   Read long                    - not supported

---     B   Write long                   - not supported

---     C   Seek

---     D   Alternate disk reset

---     E   Read sector buffer

---     F   Write sector buffer

---     10  Test drive ready

---     11  Recalibrate

---     12  RAM diagnostic

---     13  Drive diagnostic              - not supported

---     14  Controller internal diagnostic- not supported

Extended ROM BIOS Functions for Fixed Disk

The following registers are used by the absolute commands;

AL                 Number of sectors

DL                 Absolute drive number (80h for

internal and 81h for external)

CX                 Logical sector number (0-FFFFh)

ES:BX              Address of memory buffer to be

used for the operation.

---     20   Absolute read sector

---     21   Absolute write sector

---     22   Absolute format track good

---     23   Absolute format track bad

---     24   Absolute read long

---     25   Absolute write long

.pa�

VI ROM BIOS INTERFACE ROUTINES

----------------------------------------------------------------

13 continued------Disk I/O--------------------------------------

----------------------------------------------------------------

Intr   Func    In      Out       Comments

(AH)    Regs    Regs

Extended ROM BIOS Functions for Floppy Drives

---     86  Get diskette type

AL       Diskette type-returned values;

00 Disk in drive is MFM

01 Disk in drive is GCR

---     87  Get drive status

AL       Status byte-returned values;

80 Write protected

40 Door is open

20 Door state has changed

10 Motor up to speed

---     88  Read desired GCR sectors

---     89  Write desired GCR sectors

---     8A  Verify desired GCR sectors

---     8B  Determine number of drives

---     8C  Track DC erase

----------------------------------------------------------------

14-------------------RS232 I/O----------------------------------

----------------------------------------------------------------

Int     Func   In        Out      Comments

(AH)   Regs      Regs

---     0   Initialize Communications Port

DX                 Interface card (0=com1,1=com2)

AL                 Parameters to set up;Bits are

as follows;

A L

0 0 0 0 0 0 0 0

--- - --- -----

1  2  3    4

1 Word length 10=7bit,11=8bit

2 Stopbits 0=1,1=2

3 Parity 00=none,01=odd 11=even

4 Baud 000=110,001=150,010=300

011=600,100=1200,101=2400

110=4800,111=9600

---     1   Send Character to Communications Line

DX                 Interface card 0=com1 1=com2

AL                 Character to send

AH         Set as in status (call 3)

---     2   Receive Character from Communications Line

DX                 Interface card 0=com1 1=com2

AL         Character

AH         0 if no error, set as status if

non zero

.pa�

VI ROM BIOS INTERFACE ROUTINES

----------------------------------------------------------------

14 continued--------------RS232 I/O-----------------------------

----------------------------------------------------------------

Intr     Func   In      Out       Comments

(AH)   Regs    Regs

---      3   Get Communications Status

DX                Interface card 0=com1; 1=com2

AH        Line status;bits set as follows

80  Time out

40  Transmit shift reg empty

20  Transmit holding reg empty

10  Break detect

08  Framing error

04  Parity error

02  Overrun error

01  Data ready

A̠      Mode�status;bits set as follow

80  Received line signal detect

40  Ring indicator

20  Data set ready

10  Clear to send

0�� Delta received line signal

detect

04  Trailing edge ring detector

02  Delta data set ready

01  Delta clear to send

----------------------------------------------------------------

16------------Keyboard I/O--------------------------------------

----------------------------------------------------------------

Intr   Func    In      Out        Comments

(AH)    Regs    Regs

---    0    Read Next Charcter

AL         Character struck

AH         Scan code of key

---    1    Check for next character

ZF         1 if no key available, 0 if key

available

AL,AȠ��   Character/Scan code if avail.

Keystroke remains in buffer.

---    2    Get Current Shift Status

AL         Shift flag status;bits set as

follows;

80 Insert state

40 Caps lock state

20 Num lock state

10 Scroll lock state

08 Alt shift is depressed

04 Ctl shift is depressed

02 Left shift is depressed

01 Right shift is depressed

.pa�

VI ROM BIOS INTERFACE ROUTINES

----------------------------------------------------------------

17-----------Printer I/O----------------------------------------

----------------------------------------------------------------

Intr   Func    In      Out        Comments

(AH)    Regs    Regs

---    0     Print Character

DX                 Printer to be used (0,1,2)

AL                 Character to print

AH         Status set as in call 2

---    1     Initialize Printer Port

DX                 Printer to be used (0,1,2)

AH         Status set as in call 2

---    2     Get Printer Status

DX                 Printer to be used (0,1,2)

AH         Printer status,bits set as

follows;

80 not busy (ready)

40 Acknowledge

20 Out of paper

10 Selected

08 I/O error

01 Time out

----------------------------------------------------------------

1A-----------Rios Time of Day-----------------------------------

----------------------------------------------------------------

Intr   Func    In      Out         Comments

(AH)    Regs    Regs

---    0    Read Present Timer Value

AL          0 if timer has not passed 24

hours since last time read

CX          High word of timer count

DX          Low word of timer count

---    1    Set Timer Registers

CX                  High word of count to load

DX                  Low word of count to load

.pa�#